Department of Electrical Engineering

Faculty Member:      Kiran Liaqat                                    Dated:   14/03/2021    

Semester:                     2nd                                         Section:      BEE-12C      

EE-211: Electric Network Analysis

Lab 3: Sinusoidal Response Analysis and Simulation

| PLO4/CLO4 |  | PLO5/CLO5 | PLO8/CLO6 | PLO9/CLO7 |
| --- | --- | --- | --- | --- |
| Name | Reg. No | Viva /Quiz / Lab Performance 5 marks | Analysis of data in Lab Report 5 marks | Modern Tool Usage 5 marks | Ethics and Safety 5 marks | Individual and Team Work 5 marks |
| Muhammad Umer | 345834 |  |  |  |  |  |
| Saad Bakhtiar | 341150 |  |  |  |  |  |

Introduction:

In this lab, we will be simulating and generating Sinusoidal functions and using respective equipment to observe their affects. We will also learn how to observe/analyze graphs on Oscilloscope and use tracking cursor to find the (x, y) values of a generated function.

Objective:

After performing this experiment, students will be able to:

- Analyze Sinusoidal Graphs

- Set up inductive/capacitive circuit and observe their graphs using the Oscilloscope

- Further strengthen their concepts on Transients

Equipment:

- Digital Oscilloscope

- Digital Function generator

- Circuit Board Set Up

- Resistor, Capacitor and Inductor

Software:

- PSpice

Conduct of Lab

The students are required to work in groups of three to four; each student must attempt to understand and use the laboratory set-up and conduct at least one or two parts of the requirement experimentation.  The lab attendants and Lab Engineer will be available to assist the students.

In case some aspect of the lab experiment is not understood the students are advised to seek help from the teacher, the lab attendant or the assigned Lab Engineer (LE).

Analysis of RL circuit to Sinusoidal Response

## Insights:

- The inductor voltage leads the source voltage

- Inductance causes a phase shift between voltage and current that depends on the relative values of the resistance and the inductive reactance

- The phase angle is the phase difference between the total current and the source voltage

- Impedance magnitude: Z = √ R2+ X2

- Phase angle: θ = tan-1(XL/R)

## Theoretical Work:

- VL(t) = 355.75 cos (4000πt + 69.160) mV

- IL (t) = 28.31 cos (4000πt + -20.840) mA

- Phase angle of VL is 69.160 and VM L is 355.75 mV

- Phase angle of IL is -20.84 and IM is 28.31 mA

Simulation:

- Phase angle of VL is 360°×f×∆t=360°×2.k×96.u=69.120

- Value of VM 1 is 1 V

- Value of VM L is 355.868 mV

- Phase angle of IL is -90°×θVL= -20.88°

- Value of IM is 28.3 Ma

Oscilloscope Display:

Analysis of RC circuit to Sinusoidal Response

## Insight:

- The capacitor voltage lags the source voltage

- Capacitance causes a phase shift between voltage and current that depends on the relative values of the resistance and the capacitive reactance

- Phase angle: θ = tan-1(XC/R)

## Theoretical Work:

- VC(t) = 868.8 cos (4000πt – 64.250) mV

- IC (t) = 54.58 cos (4000πt + 25.750) mA

- Phase angle of VC is -64.250 and VM C is 868.8 mV

- Phase angle of IC is 25.750 and IM is 54.58 mA

Simulation:

- Phase angle of VC is -360°×f×∆t=-(360°×2.k×89.502.u)=-64.440

- Value of VM 1 is 2 V

- Value of VM C is 867.3 mV

- Phase angle of IL is 90°+θVC=25.56°

- Value of IM is 54.6 mA

Oscilloscope Display:

Conclusion:

After performing this lab, we have learnt;

- Analyzing sinusoidal waveforms of RL/RC circuits

- Set up capacitive/inductive circuits on breadboard through means of Function Generator and analyzed through Oscilloscope