Department of Electrical Engineering

Faculty Member: Arshad Nazir	Dated: 26/11/2021

Semester:      3rd      	Section: BEE 12C

Group No.: 7

EE-221: Digital Logic Design

Lab 9: XS – 3 To Gray Code Using NAND Gates

|  |  | PLO4/CLO4 | PLO4/CLO4 | PLO5/CLO5 | PLO8/CLO6 | PLO9/CLO7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Reg. No | Viva / Lab Performance | Analysis of data in Lab Report | Modern Tool Usage | Ethics and Safety | Individual and Teamwork | Total marks Obtained |
|  |  | 5 Marks | 5 Marks | 5 Marks | 5 Marks | 5 Marks | 25 Marks |
| Muhammad Adil Azeem | 342459 |  |  |  |  |  |  |
| Muhammad Umer | 345834 |  |  |  |  |  |  |
| Raahim Ahmad Waqar | 342287 |  |  |  |  |  |  |
| Saad Bakhtiar | 341150 |  |  |  |  |  |  |
| Syed Aun Ali Kazmi | 342384 |  |  |  |  |  |  |

## Lab 9: XS – 3 To Gray Code

This Open-ended Lab has been divided into two parts:

In first part you are required to design and implement a Excess-3 to gray code converter.

The next part is the Verilog Modeling and Simulation of the Circuit you implemented in you first part.

Objectives:

- Understand steps involved in design of combinational circuits

- Understand binary codes for decimals and their hardware realization

- Write code for combinational circuits using Verilog Gate Level Modeling

- Design a circuit in Verilog by calling different modules

Lab Instructions

- This lab activity comprises three parts, namely Pre-lab, Lab tasks, and Post-Lab Viva session.

- The lab report will be uploaded on LMS three days before scheduled lab date. The students will get hard copy of lab report, complete the Pre-lab task before coming to the lab and deposit it with teacher/lab engineer for necessary evaluation. Alternately each group to upload completed lab report on LMS for grading.

- The students will start lab task and demonstrate design steps separately for stepwise evaluation (course instructor/lab engineer will sign each step after ascertaining functional verification)

- Remember that a neat logic diagram with pins numbered coupled with nicely patched circuit will simplify trouble-shooting process.

- After the lab, students are expected to unwire the circuit and deposit back components before leaving.

- The students will complete lab task and submit complete report to Lab Engineer before leaving lab.

- The Total duration for the lab is 3 hrs. After lab duration, a deduction of 5 marks per day will be done for late submission.

- A lab with in-complete lab tasks will not be accepted.

- There are related questions at the end of this activity. Give complete answers.

Pre – Lab Tasks

Pre – Lab Task 1

In the lab you would be implementing an Excess-3 to gray code converter. Make a truth table for both the codes by filling in the following tables and simplify the expressions for W, X, Y, Z in terms of A, B, C, D. (Use backside of the page if necessary).

XS - 3 to Gray Converter

| Dec | XS - 3 |  |  |  | Gray Code |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | W | X | Y | Z |
|  | 0 | 0 | 0 | 0 | X | X | X | X |
|  | 0 | 0 | 0 | 1 | X | X | X | X |
|  | 0 | 0 | 1 | 0 | X | X | X | X |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 3 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 4 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 5 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 6 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| 7 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 8 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 9 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
|  | 1 | 1 | 0 | 1 | X | X | X | X |
|  | 1 | 1 | 1 | 0 | X | X | X | X |
|  | 1 | 1 | 1 | 1 | X | X | X | X |

Karnaugh Maps for Simplified Expressions

Simplified Expressions

| W | AB + ACD | A (B + CD) |
| --- | --- | --- |
| X | A + BCD | A + B (CD) |
| Y | B’C’D’ + BD + BC | B’C’D’ + B (C + D) |
| Z | C’ | C’ |

Pre – Lab Task 2

Draw the logic diagram for Excess-3 to gray code converter using AND, OR and NOT gates in the space provided below. You can use 2, 3, 4 input gates if required.

Pre – Lab Task 3

Draw the logic diagram for Excess-3 to gray code converter using only NAND gates in the space provided below, you can use 2, 3, 4 input Nand gates if required.

Pre – Lab Task 4

Draw the logic diagram for Excess-3 to gray code converter using only NOR gates in the space provided below, you can use 2,3,4 input Nor gates if required.

Lab Tasks

Lab Task 1

Implement Excess-3 to gray code converter using only NAND gates on hardware. Paste the complete circuit diagram, depicting hardware results.

| Dec | XS - 3 |  |  |  | Gray Code (Hardware) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | W | X | Y | Z |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 3 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 4 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 5 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 6 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| 7 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 8 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 9 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |

Lab Task 2

Design and simulate the circuit K – Map equations you obtained in Pre – Lab task 1 in Verilog dataflow modeling. Give the code and testbench and waveform in the space provided below.

Verilog Code

module xs3_to_gray(w, x, y, z, a, b, c, d);

input a, b, c, d;

output w, x, y, z;

assign w = a & (b | (c & d));

assign x = a | (b & (c & d));

assign y = (~b & ~c & ~d) | (b & (c | d));

assign z = ~c;

endmodule

module testbench();

reg A, B, C, D;

wire W, X, Y, Z;

xs3_to_gray t1(W, X, Y, Z, A, B, C, D);

initial begin

#100 A = 0; B = 0; C = 1; D = 1;

#100 A = 0; B = 1; C = 0; D = 0;

#100 A = 0; B = 1; C = 0; D = 1;

#100 A = 0; B = 1; C = 1; D = 0;

#100 A = 0; B = 1; C = 1; D = 1;

#100 A = 1; B = 0; C = 0; D = 0;

#100 A = 1; B = 0; C = 0; D = 1;

#100 A = 1; B = 0; C = 1; D = 0;

#100 A = 1; B = 0; C = 1; D = 1;

#100 A = 1; B = 1; C = 0; D = 0;

end

endmodule