NATIONAL UNIVERSITY OF SCIENCES & TECHNOLOGY

Instrumentation and Measurements (EE-383)

Assignment # 1

| Submitted to: | Dr. Shahzad Younis |
| --- | --- |
| Submitted by: | Muhammad Umer |
| Class: | BEE-12C |
| Semester: | 5th |
| Dated: | 13/10/2022 |
| CMS ID: | 345834 |

Question: Identify digital (05) and analog (05) sensors which could be interfaced with micro-controller. Write main features of the sensors, along with block diagram/picture and pin configuration for each. Moreover make a table to list all the parameters i.e., range, accuracy, precision, min, max, average values etc. Make use of datasheets to get the information.

# Digital Sensors

## Compact Absolute Pressure Sensor - MPL3115A2

- Main Features

- Programmable interrupts

- Autonomous data acquisition

- Embedded 32-sample FIFO

- Data logging up to 12 days using the FIFO

- One-second to nine-hour data acquisition rate

- I2C digital output interface

- Fully compensated internally

- Direct reading

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VDD | Power supply |
| CAP | External capacitor |
| GND | Ground |
| VDDIO | Digital interface supply |
| INT2 | Pressure interrupt 2 |
| INT1 | Pressure interrupt 1 |
| SDL | I2C Serial Data |
| SCL | I2C Serial Clock |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Calibrated range | 70 - 150 kPa |
| Calibrated temperature output | -40 °C to 85 °C |
| Operating range | 50 - 150 kPa |
| Operating temperature | -40 °C to 85 °C |
| Max pressure | 500 kPa |
| Absolute accuracy | ± 0.4 kPa |

## Programmable Resolution 1-Wire Digital Thermometer - DS18B20

- Main Features

- Unique 1-Wire Interface Requires

- Reduce Component Count with Integrated Temperature Sensor and EEPROM

- Parasitic Power Mode Requires Only 2 Pins for Operation

- Simplifies Distributed Temperature-Sensing Applications with Multidrop Capability

- Flexible User-Definable Nonvolatile (NV) Alarm Settings

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VDD | Power supply |
| DQ | Data Input/Output |
| GND | Ground |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Thermometer error | ± 2 °C |
| Accuracy | ± 0.5 °C |
| Temperature resolution | 9 to 12 bits |
| Operating temperature | -55 °C to 125 °C |
| DQ input current | ± 0.4 kPa |
| Conversion time | < 750 ms |

## Ambient Light Sensor - BH1750

- Main Features

- I2C bus interface

- Spectral responsibility is approximately human eye response

- Illuminance to digital converter

- Supports continuous measurement mode

- Supports one-time measurement mode

- Low current by power down function

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VCC | Power supply |
| GND | Ground |
| SCL | SCL pin for I2C communication |
| SDA (Data) | SDA pin for I2C communication |
| ADD* | Selects address |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Resolution modes | High, H-2, Low |
| Accuracy | ± 1.4% cd. |
| Operating temperature | -40 °C to 85 °C |
| Peak wave length | 560 nm |
| Powerdown current | 1 μA |
| Conversion Time | < 750 ms |

## Distance Measuring Sensor Unit - GP2Y0D810Z0F

- Main Features

- Short distance type

- Sunlight tolerance

- Battery drive possible

- Low profile; weight without header pins: 1.3 g

- Add VIN terminal, and an external transistor of VCC line is unnecessary at operation

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VCC | Power supply |
| GND | Ground |
| DO | Digital output |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -10 °C to 60 °C |
| Measuring duration | 2.56 ms |
| Supply current | 5 mA |
| Detecting distance | 20 - 100 mm |

## Digital IR Sensor - TSOP38238

- Main Features

- Adjustable Sensing range

- Built-in Ambient Light Sensor

- Mounting hole

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VCC | Power supply |
| GND | Ground |
| DO | Digital output |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -20 °C to 50 °C |
| Supply current | 20 mA |
| Obstacle detection range | upto 20 cm |

# Analog Sensors

## Analog Temperature Sensor - HX710A

- Main Features

- On-chip temperature measurement

- On-chip power-on-reset

- On-chip oscillator requiring no external component

- DVDD and AVDD supply voltage difference measurement

- Simultaneous 50 and 60Hz supply rejection

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VCC | Power supply |
| GND | Ground |
| DOUT | Analog output |
| SCK | Clock |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -40 °C to 85 °C |
| Supply current | 1100 μA |
| Temperature drift | ± 15 nV/°C |
| Output data rate | 10/40 Hz |
| Power supply rejection | 100 dB |
| Output settling time | 400 ms |

## Analog Water Pressure Sensor – DFRobot Gravity

- Main Features

- Support liquid level detection in special situation

- Support water pressure detection of tanks

- Support water pressure detection of outdoor environment

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VS | Power supply |
| GND | Ground |
| CS | Chip select |
| SDATA | Serial analog data |
| SCLK | Clock |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -20 °C to 85 °C |
| Quiescent current | 2.8 mA |
| Temperature drift | ± 15 nV/°C |
| Response time | <2 ms |
| Accuracy | 0.5 % |
| Operating pressure | < 2 MPa |

## Pressure Transducer - MS4515

- Main Features

- Inches H2O Pressure Ranges

- PCB Mountable

- High Level Analog Output

- Barbed Pressure Ports

- Hosts 1/8” barbed pressure ports

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| SUPPLY | Power supply |
| GND | Ground |
| OUTPUT | Analog output |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -10 °C to 85 °C |
| Supply current | 3 mA |
| Creep time | 6 ms |
| Response time | 1 ms |
| Accuracy | 0.5 % |
| Output voltagemax | 4.75 V |

## Precision Centigrade Temperature Sensor - LM35

- Main Features

- Calibrated Directly in Celsius

- Low-Cost Due to Wafer-Level Trimming

- Low-Impedance Output

- Low Self-Heating

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VS | Power supply |
| GND | Ground |
| OUTPUT | Analog output |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -55 °C to 150 °C |
| Quiescent current | 60 μA |
| Temperature rating | -55 °C to 150 °C |
| Nonlinearity | ± 0.18 °C |
| Accuracy | ± 0.4 °C |

## Transformer Drive Ultrasonic Sensor - TUSS4440

- Main Features

- Integrated driver for transformer driven transducers and receiver stage with analog output

- Configurable drive stage

- Serial Peripheral Interface (SPI) for configuration by microcontroller

- 86-dB input dynamic range analog front-end

- Block Diagram

- Pin Configuration

| Pin | Description |
| --- | --- |
| VPWR/VDD | Power supply |
| GND | Ground |
| INN | Negative transducer receive |
| INP | Positive transducer receive |
| SDI | SPI data input |
| SDO | SPI data output |
| OUT A/B | Transducer driver output |

- Parameter Table

| Parameters | Values |
| --- | --- |
| Operating temperature | -40 °C to 105 °C |
| Maximum current rating | 25 mA |
| Maximum output load capacitance | 10 pF |
| Input hysteresis | 100 mV |
| Supported transducer frequencies | 40 - 400 kHz |
| Dynamic input range | 86 dB |